三色LED  RGB模块

使用三色全彩LED制造

模块有3个输出：

1. R，红色输出,

2. G，绿色输出,

3. B，蓝色输出.

模块特点：

3组信号输出，可通过单片机编程实现R，G，B三种颜色的混合达到全彩的效果，

实验代码:

int ledPin = 13; // LED is connected to digital pin 13

int redPin = 11;  // R petal on RGB LED module connected to digital pin 11

int greenPin = 9;  // G petal on RGB LED module connected to digital pin 9

int bluePin = 10;  // B petal on RGB LED module connected to digital pin 10

void setup()

{

pinMode(ledPin, OUTPUT); // sets the ledPin to be an output

pinMode(redPin, OUTPUT); // sets the redPin to be an output

pinMode(greenPin, OUTPUT); // sets the greenPin to be an output

pinMode(bluePin, OUTPUT); // sets the bluePin to be an output

}

void loop()  // run over and over again

{

// Basic colors:

color(255, 0, 0); // turn the RGB LED red

delay(1000); // delay for 1 second

color(0,255, 0); // turn the RGB LED green

delay(1000); // delay for 1 second

color(0, 0, 255); // turn the RGB LED blue

delay(1000); // delay for 1 second

// Example blended colors:

color(255,255,0); // turn the RGB LED yellow

delay(1000); // delay for 1 second

color(255,255,255); // turn the RGB LED white

delay(1000); // delay for 1 second

color(128,0,255); // turn the RGB LED purple

delay(1000); // delay for 1 second

color(0,0,0); // turn the RGB LED off

delay(1000); // delay for 1 second

}

void color (unsigned char red, unsigned char green, unsigned char blue)     // the color generating function

{

analogWrite(redPin, 255-red);

analogWrite(bluePin, 255-blue);

analogWrite(greenPin, 255-green);

}